Commonly Used Matlab Functions:

- roots() - Finds the roots (solutions) of a polynomial equation, often used to determine system poles and zeros.

- tf() - Creates a transfer function model from numerator and denominator coefficients, representing linear systems in Laplace domain.

- zpk() - Constructs a zero-pole-gain model, specifying a system by its zeros, poles, and gain.

- step() - Simulates and plots the time response of a system to a unit step input.

- stepinfo() - Returns performance metrics such as rise time, settling time, and overshoot for the step response of a dynamic system.

- impulse() - Simulates and plots the time response of a system to a unit impulse input.

- residue() - Computes the partial fraction expansion (residues, poles, and direct terms) of a rational function, useful for inverse Laplace transforms.

- s=tf('s') - Defines the Laplace variable 's' as a transfer function object, enabling symbolic manipulation of transfer functions.

- rlocus() - Plots the root locus of a control system, showing how the poles of the closed-loop transfer function vary as a system parameter (typically gain) changes.

- feedback() - Computes the closed-loop transfer function of a system with feedback by connecting two systems in a feedback configuration.

- plot() - Creates two-dimensional line plots of data, allowing visualization of the relationship between variables.

- bode() - Generates the Bode plot, displaying the frequency response (magnitude and phase) of a linear system.

- margin() -  Plots the Nyquist diagram, showing the frequency response of a system in the complex plane to assess stability.

- nyquist() - Calculates and displays the gain margin, phase margin, and crossover frequencies of a system’s frequency response.